User Manual

PON Optical Power Meter

Version：V3.0

Contents

Notes	2

1.Standard Configuration	3

2.Overview	3

3.Technical Specifications	4

4.Instruction	7

5.Trouble shooting	21

6.Maitanance	22

7.Warranty & Servicing	22

Appendix I	24

Notes

Icon Meaning：

In this manual will appear some text with the below icon，The meaning as follows：

Warning：Improper behavior and operation, to prevent any wrongdoing and harm.

Note：Due to the improper operation may cause harm to the human body or product, so special note to warn the user.Please make sure that!

Important note：To identify the key information, general notes, noun explanation or reference information, etc.

WEEE Sigh：This product or component part (s) shall not be considered as a general garbage dump, please according to your area on waste treatment method for disposal of such products.

About Battery

This series of products are different types used in power supply or have a different way. There are disposable alkaline batteries, or rechargeable batteries. Do not mixture of different types or different capacity of battery, prohibited to charge a rechargeable battery.

About Storage

When product long-term deposit or do not use, please remove products within the battery stored separately, in order to avoid battery leakage caused by instrument damage.

To prevent electric shock, please do not open the product case, must be the company authorized by a qualified professional personnel for repair; Do not expose this product in the rain or damp environment, so as to avoid fire or click on the occurrence of danger!

Due to the laser can cause the big harm to eyes,please do not look directly into the laser output when you use the tester.

Chapter 1 Standard Configuration

Please refer to the following models corresponding to the standard configuration, check whether your product accessories is complete, if lack of any items, please contact the dealer as soon as possible!

| Model | Pon Optical Power Meter |  |
| --- | --- | --- |
| Items | Title | Quantity |
| 1 | tester | 1unit |
| 2 | User Manual | 1pc |
| 3 | USB | 1pc |
| 4 | Soft CD | 1pc |
| 5 | 1.5VAA Battery | 3pc |

Chapter 2 Overview

PON Optical Power Meter target at the FTTx application and maintenance. This power meter is able to simultaneously test and estimate the signals of the voice, data and video. It is an essential and ideal tool for the construction and maintenance of the PON projects.

Features：

(1).It can experiment at Voice, data and video signal synchronous measurement and display on BPON/EPON/GPON.

(2)Providing simultaneous measurement for all three wavelengths on the fiber (1490nm, 1550nm,1310nm )

⑶Used in Burst mode measurement of 1310nm upstream.

⑷．Use the software connect with PC, setting the threshold, data transfer, and calibration the wavelength.

⑸．USB communication port enables data transfer to a PC.1000measurement items can be saved in PON power meter or computer for data review.

⑹．With optical power meter module,include850、1300、1310、1490、1550、1625sixs(AP，Awithout850nmwavelength）；With visual fault locator module

（PON and AV）

⑺．Optical power meter and VFL with one port.（only A）

⑻．Optional Chinese/English display.

⑼．Offers up to 10 different threshold sets in total,Three status LEDs represent different optical signal conditions of Pass, Warn and Fail respectively.

⑽．10 minutes Auto-off function can be activated or deactivated

⑾．Good key design,high sensitivity, greatly reducing the volume and weight of the tester.

⑿．Different models corresponding to different function, according to  own use to choose .

Chapter 3 Technical Specifications

3.1 PON module:

| PON module: | PON Optical Power Meter |  | A | AV | AP |
| --- | --- | --- | --- | --- | --- |
| 1310 upstream measurement |  |  |  |  |  |
| Pass Zone(nm) | 1260nm～1360nm |  |  |  |  |
| MeasurementRange(dBm) | -40dBm～+10dBm |  |  |  |  |
| Output power(max) | 15dBm |  |  |  |  |
| Isolation@1490/1550(dB) | >40dB |  |  |  |  |
| Burst mode measurement error | <±0.5dB |  |  |  |  |
| 1490 downstream measurement |  |  |  |  |  |
| Pass Zone(nm) | 1475nm～1505nm |  |  |  |  |
| MeasurementRange(dBm) | -40dBm～+10dBm |  |  |  |  |
| Output power(max) | 15dBm |  |  |  |  |
| Isolation@1310/1550(dB) | >40dB |  |  |  |  |
| 1550 downstream measurement |  |  |  |  |  |
| Pass Zone(nm) | 1535nm～1565nm |  |  |  |  |
| MeasurementRange(dBm) | -40dBm～+20dBm |  |  |  |  |
| Output power(max) | 25dBm |  |  |  |  |
| Isolation@（1310/1490nm） | >40dB |  |  |  |  |
| Measurement Accuracy |  |  |  |  |  |
| Connatural uncertainty(dB) | ±0.5dB |  |  |  |  |
| Linearity(dB) | ±0.1dB |  |  |  |  |
| Passing through insertion Loss(dB) | <1.5dB |  |  |  |  |
| General Information |  |  |  |  |  |
| Detector Type | InGaAs |  |  |  |  |
| Optical Connector | FC/SC/ST Interchangeable/2.5 universal adapter |  |  |  |  |
| Fiber Type | SM 9/125um |  |  |  |  |
| Measurement Unit | dB/dBm/xW |  |  |  |  |
| Resolution (dB) | 0.01dB |  |  |  |  |
| Operation Voltage(V) | DC 3.3V～5.5V |  |  |  |  |
| Power Supply | 3pc1.5V battery |  |  |  |  |
| Continuously Operation time (h) | PON：90h | PON：90h OPM：100h VFL：50h |  | PON：90h VFL：50h | PON：90h OPM：100h |
| Operation Temperature(℃) | -10℃～60℃ |  |  |  |  |
| Storage temperature(℃) | -25℃～70℃ |  |  |  |  |
| Weight(kg) | 423g | 425g |  | 424g | 424g |

Note: The operation time of the battery are all for the instrument that do not turn on backlight, if the backlight turn on the operation time will be shorted.

3.2 Normal Optical Power Meter Module：：

| Normal Optical Power Meter | PON Optical Power Meter② | A | AP |  |
| --- | --- | --- | --- | --- |
| Measurement Accuracy |  |  |  |  |
| Connatural uncertainty(dB) | None | ±0.5dB |  |  |
| Linearity(dB) |  | ±0.1dB |  |  |
| Measurement Range(dBm) |  | -70dBm～+6dBm |  |  |
| General Information |  |  |  |  |
| Measurement Unit | None | dB/dBm |  |  |
| Resolution (dB) |  | 0.01dB |  |  |
| Calibration Wavelength(nm) |  | 1310/ 1490/1550/1625 |  | 850/1300/1310/ 1490/1550/1625 |
| Detector Type |  | InGaAs |  |  |
| Optical Connector |  | FC/SC/ST Interchangeable/2.5 universal adapter |  |  |

②：PON Optical Power Meter do not have the OPM module

3.3VFL Module：

| VFL | PON Optical Power Meter③ | A | AV |
| --- | --- | --- | --- |
| Output power | None | >1mW |  |
| Wavelength |  | 650nm |  |
| Optical Connector |  | FC/SC/STInterchangeable/2.5 universal adapter |  |
| Fiber Type |  | SM/MM |  |

③：PON Optical Power Meter without VFL module.

Chapter 4 Instruction

PON Optical Power Meter / A two models are different between functions and the form, detailed in this chapter, please check carefully.

1

3

2

1.Describtion

13

12

10

9

11

8

5

7

14

6

4

1- ONT：1310nm Up Stream Port（1310nm）

2- OLT/VIDEO：1490nm/1550nm Down Stream

3- VFL or OPM Port

4- Display

5- ONT1310nm Up Stream Port LEDs

6- OLT1490nm/1550nm Down Stream LEDs

7- Video signal LEDs (1550nm)

8- AUTO power/off key

9-UPIn the first interface, switch visual fault location model（VFL）

10- ENTER: Confirmation the function selection and saving the data.

11- MENU: PON/CW Power meter module Option

12- DOWM.In the first interface,Switch betweenVFL in continuous light (the CW) and pulse light (HZ)

13- CANCEL:Function cancel

14- USB PORT

④：Only A and AV up/down keys for VFL function

PON Online test connection diagram

Data Networks Analyser IXIA/Smartbits

Optical Splitter

OLT

Fiber

……

Fiber

ONT

OLT PON Interface optical power test configuration diagram

Data Networks Analyser IXIA/Smartbits

ONT

Variable Optical Attenuator

Optical Splitter

……

1490nm

OLT

1310nm

Fiber

Lamda Multiplexer

ONT The sensitivity test configuration diagram

Data Communication

Run the CD,install USB driver software CP210x_VCP_Win2K_XP_S2K3.exe .

or

Then install dotNetFx40_Full_x86_x64.exe.

Connect PON power meter to the computer’s port using the provided USB cable,Open the software as shown in below figure:

Upload the data

（1）Open the PON power meter，first of all click “connect”，connect PON power meter to the computer(Click“English”to set the language of the operation interface to be English )

（2）choose “Upload Data”，click “Upload Test Data”sign，the saved data can be read on the computer, and saved as a file format.

（3）click the“Delete Test Data” button or“Clear Test Data”to delete the data.

Threshold Setting：

Click“Upload threshold value”icon to read the threshold.

From the toolbar, select “Threshold Set”, and right click the Threshold to type or modify the threshold value.

Calibration

Choose “Calibration” tab, the user can perform user self-calibration operation also can choose “Factory Reset” mode. For example, the current optical power at 1310nm is -10dBm, type -10dBm in the text box of input standard power, and click Calibration tab, then finished the calibration operation at 1310nm wavelength.

Power ON/OFF

1.Power ON

Press  for about two seconds to make the unit power on，Enter into the PON power meter function , the unit displays as shown in below figure:

As shown in above figure, the LCD will display simultaneously three wavelengths of 1310nm upstream measurement and 1490nm,1550nm downstream measurements. When optical signal is present under test, the optical channel will display power levels in dBm, when optical signal is not available, then the optical channel will display “LOW”. “”indicates 10 minutes auto-off function is activated.The unit will turn itself off after 10 minutes of idle time. Press  to close 10 minutes auto-off.

2.Power Off

Press  key for about two seconds,to turn off the unit.

PON Power Meter module

PON power meter can simultaneously measure the PON network upstream signals of 1310 nm, 1490 nm downstream data signal and descending video signal output power of 1550 nm

After open the instrument, Pressto enter into menu operation, Press “” to exit

Press“”or“”to choose the following settings, and press “” to choose the function.

Use“or ” to choose the required  threshold value,(The thresholds rang and name have already set in the software, and also download to the instrument, please check with the user manual of Chapter 6 data communication), For example choose threshold and name it as “a” and press“”.

Press“” back to the testing menu.

Clean the tested optic cable, connect with OLT/VIDEO port and ONT port, (SC type connector), noted with the optic adaptor type, the optical adaptor will be break, and the tested value will be faulty if the un-matched adaptor connected

Pay attention to the type of the measured optical fiber connectors, does not match the type of access instrument, may damage the instrument output optical fiber connector,and can not get the true result.

1310nm,upstream, 1490nm, 1550nm downstream, and 3 wavelength measurement at the same time.

Eg.: The selected threshold name is “a”(-10dBm,-20dBm,-30dBm). The measured result displayed: -11dBm (Optical power) at 1310nm upstream measurements shown in above figure, as it is within -10dBm (Pass threshold)~-20dBm(Warning threshold), therefore, it means the optical signal in Pass status and the LED at 1310nm will light up with green color: If the measured result is between -20~-30dBm, it means the optical signal in Warn status and the LED at 1310nm will light up with yellow color which means the optical signal may got problems but still can be used; If the measured result is out of the range of thresholds, i.e greater than -10dBm or smaller than -30dBm, then it means the optical signal got problems, the LED at 1310nm will light up with red color. The same process be used at 1490 and 1550nm wavelengths.

Press“”, there will displaying saving on the screen, check with the upper pic. The data recording series No.is 0002. press“” saving, and “” for cancel, and the saving recording can be checked in the “data view” under “historical data” Menu.

After finish the testing work, keep the connect port  clean, so cover the dusty-cap once finish the work.

Instructions for Normal Optical Power Meter Function⑤

Press  button to open the instrument, use “” button and ; to choose the Optical Power Meter measurement, then press “” to enter into the selected function. orButton in this case, is to choose the wavelength. Instrument only provide the test of 850nm、1300nm、1310nm、1490nm、1550nm、1625nm,display “-70dBm”when without light.

Take 1310nm measurement for example:

Connect with the tested optic fiber

Clean the optic Fiber end-face, and connect with the PIN port on the instrument (SC/ FC/ST adaptor), noted with the type of the optic fiber adaptor, the non-matched connecting will make the damage of the connector on the instrument and also make the faulty of the measuring result.

E.g.: 1310nm measurement, after connect with the optic fiber, it will displaying -20dBm, press“”, set the -20dBm as the reference value, and it will also display relative value (dB), see as following:

Ref

In order to get the loss between the two measurements, do another time testing under the first relative output power. The relative value will be calculated automatically. For example the second time testing of 1310nm output power is -35dBm, the relative output power will be -15dBm.(relative output power=absolute output power-reference value -15dBm=-35.00dBm-（-20.00dBm） see as following:

Absolute value

Relative value

After finishing testing, cover the dusty cap to keep the optical connector port to be clean.

⑤：PON Optical Power Meter &AV without this function.

VFL module⑥

During the event or break point checking of the short distance optic fiber and pigtails we can use the VFL function.

First of all, Clean the optic Fiber end-face, and connect it them the VFL port (FC adaptor), after open the instruments, press “” to activate this function, then can see the visual laser at the breaking point or the end of the pigtails. Press “” to shift the CW and the Hz wavelength. And press again to close this function.

⑥：PON Optical Power Meter &AP without this function.

Other function instruction:

PON Power Meter:

Relative output power measurement:

As per requirements of measurement to select the different wavelengths and Ref settings, relative power=testing value-Ref value. Press  or to select the different wavelengths, Press“” to change the Ref value as shown in below figure

REF

RELATIVE VALUE

Threshold Selection

It offers up to 10 different threshold sets in total. There are three status LEDs in the front panel and each light indicator has three conditions of colors which represent different signal status as below:

Green---Represents Pass

Yellow---Represents Warning

Red----Represents Fail

Behind“ThreshSel”displays selected threshold setting order number from 1~10, and behind “Name”displays selected threshold name, press orto select the corresponding shresholds. With using software to modify the values range and names of thresholds (Detailsas explained in term 5-Data Communication.)

Status

LEDs

⑦：Threshold Selection just can be use English or figure,can not use Chinese.

Switches of measurements units

Measure the output power of the fiber use dBm or Xw, Press “” with  and  to choosethe measurements units, press “” to exit and cancel the current operation as shown in below figure。

Switches of language

There are two language English and Chinese to Choose, Press “” and to select “Language” option, Press “” to exit and cancel the current operation.

Time setting

In item 5 have “ Time Setting”Press  and select the “Time Set”option, and press  to change year-month-date-time respectively as shown in below figure,use andto change the digits. After setting, press “” to confirm.Press “” to exit and cancel the current operation.

This option is designed to review and delete saved measurements data.

Press “ Menu” to enter into menu operation, using with  andto highlight “History Data” option. There are three options in History data:

- ．”Data View”

Under “History data”, use  and to highlight “Data View”, press “” to confirm and view the history data as shown in below figure. Press “” to exit.

As shown in above figure, when there is no optical signal to be tested, the power level will show -40dBm at 1310nm&1550nm and -50dBm at 1550nm, when the optical signal is present, the unit will display the corresponding tested value. “Thresh Sel 5” is the name of current selected threshold set.  0001 here represents the current viewed data, 0002 here means the total qty of saved measurements data. Use  andto choose or change the different saved history data. If there is no measurements is saved in the unit, it will display “No Record”

- ．“Del Cur”

Under “History data”, use  and to highlight “Del Cur”, press “” to confirm to delete the current data as shown in below figure.

- ．“Del All”

Under “History data”, use  andto highlight “Del All”, press “” to confirm to delete all the saved history data as shown in below figure.

Chapter 5 Troubleshooting

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| No display on the LCD | - Power Off - Lower Battery | - Press“” to turn on - Change the battery |
| Inaccurate measurements | 1.Optical interface is not clean 2. Improper fiber connection | 1Clean the fiber optic interface - Reconnect the fiber |

Chapter 6 Maintenance

To help ensure long, trouble-free operation:

- Always keep clean fiber-optic connectors before using, keep the unit free from oil and dust.

- Do not use unclean, non-standard fiber-optic connector adapter, do not insert to bad fiber-optic connector interface which may cause the system out of the work.

- Try to use one type fiber optic connector adapter.

- After finish the testing or when the unit is unused, please cover the dust-proof cap to ensure the optical interface is clean and to avoid measurements errors caused by dust.

- Push/pull the fiber-optic connectors carefully to avoid the scratches on the interface.

- Clean optical connectors regularly with cleaning cotton swabs.

Chapter 7 Warranty & Servicing

Caution: Repair it in the field is NOT recommended.

- we warrant that PON Optical Power Meter will be free from defects in material and workmanship for a period of 18 months. The date will be started from the date of goods receiving by original customer.

- If any defectives happened due to quality problems of the product during the first  month(from the date of goods receiving) of warranty period, we undertakes at its own cost (including all the freight costs and import taxes ) to repair or replace or return the faulty product.

- If any defectives happened due to quality problems of the product during the 2nd month to 18th month of warranty period, we promises to repair or replace free of charge. But the freight cost and related taxes will be shared by both parties. We will pay the shipping cost from customer side to We and pay the import taxes in China. Customer will pay the shipping cost from We to customer side and its local import taxes accordingly.

This warranty is limited to defects in workmanship and materials and does not cover damages from accident, acts of god, neglect, wrong usage or abnormal conditions of operation.

- We will charge corresponding fees for the cost of materials, repair and shipping in conditions of below:

- Defects occurred under normal use and service but out of the warranty period.

- Failures and damages occurred do not because of defects in material and workmanship of products.

- Failures and damages occurred because of failing to comply with the Operation Instruction and   necessary attention.

- abnormal conditions of operation or handling：

Such as artificial damage, or operating in abnormal conditions of like high temperature、high voltage, humidity and etc., We will charge depend on the actual failure rating.

Appendix I

WE guarantees that any information you supply will remain confidential.

By returning this card, you will automatically be notified about updates, modifications, and recalibration.

CUSTOMER SERVICE

-----------------------------------------------------------------------------------------------------

Warranty Registration Card

Serial Number:

Model Number:

Date of Purchase:

Company Name:

Company Address:

TEL:               FAX:

E-mail:

Note：Please fax this note within one month from the date of receiving units.

Do you have any comments on the quality of this product or the service from We?